# Lunar Spaceport Simulation - Class Documentation

ISRUPlant

Initialization

- system: SimPy environment instance for the simulation

- processingRate: Rate at which regolith is processed (kg/hr)

- regHeadGrade: Weight percentage of Ilmenite in the regolith feed (0-1)

- energyConsumptionRate: Base energy consumption rate for plant operations (kW)

Additional Notable Object Variables

- LOXStored: Current amount of liquid oxygen stored in the plant (kg)

- totalEnergyConsumed: Cumulative energy consumed by the plant since start (kWh)

- lg: Lunar gravity constant, 1.62 m/s² (used in energy calculations)

Functions

- processRegolith(system, regolithMass, transportDist=1): SimPy process that converts regolith into LOX over time. Takes mass of regolith to process and optional transport distance. Updates LOXStored and totalEnergyConsumed, then yields for the processing duration.

SolarPowerSystem

Initialization

- system: SimPy environment instance for the simulation

- powerOutput: Solar panel power generation rate (kW)

- batteryCapacity: Maximum energy storage capacity of batteries (kWh), default=0

- batteryDegradationFactor: Multiplier to reduce battery capacity (0-1), default=1.0

- powerDegradationFactor: Multiplier to reduce power output (0-1), default=1.0

Additional Notable Object Variables

- currentPowerOutput: Actual current power generation rate after degradation (kW)

- batteryCharge: Current energy stored in batteries (kWh)

- totalEnergyGenerated: Cumulative energy generated by solar panels (kWh)

- totalEnergyFromBattery: Cumulative energy discharged from batteries (kWh)

Functions

- generatePower(duration): Calculates energy generated over a time period (hours). Returns energy amount and updates totalEnergyGenerated.

- chargeBattery(energy): Stores excess energy in battery up to capacity limit. Returns amount actually stored.

- dischargeBattery(energy): Withdraws energy from battery. Raises RuntimeError if insufficient energy available and battery depleted.

- degradePower(factor): Applies degradation multiplier to reduce solar panel output.

- degradeBattery(factor): Applies degradation multiplier to reduce battery capacity.

PowerManager

Initialization

- system: SimPy environment instance for the simulation

- solarSystem: SolarPowerSystem object that provides power generation

Additional Notable Object Variables

- consumers: List of all registered power-consuming objects in the system

Functions

- registerConsumer(consumer): Adds a power consumer to the managed list. Consumer must have getCurrentPowerDemand() method.

- managePower(dt=1.0): SimPy process that continuously balances power generation and consumption. Automatically charges/discharges batteries and manages energy flow every dt hours.

HabitationModule

Initialization

- system: SimPy environment instance for the simulation

- name: Identifier string for the habitat module

- constantPowerRate: Continuous baseline power consumption (kW)

Additional Notable Object Variables

- totalEnergyConsumed: Cumulative energy used by the habitat (kWh)

- spikeEvents: List of scheduled one-time power events as (time, energy) tuples

Functions

- setConstantPowerRate(rate): Updates the continuous power consumption rate (kW).

- scheduleSpike(time, energy): Schedules a one-time power draw event at specified simulation time (hr) with given energy requirement (kWh).

- getCurrentPowerDemand(dt): Calculates total power demand for time period dt. Automatically processes any spike events and updates totalEnergyConsumed.

CommunicationModule

Initialization

- system: SimPy environment instance for the simulation

- name: Identifier string for the communication module

- constantPowerRate: Continuous baseline power consumption (kW)

Additional Notable Object Variables

- totalEnergyConsumed: Cumulative energy used by the communications system (kWh)

- spikeEvents: List of scheduled one-time power events as (time, energy) tuples

Functions

- setConstantPowerRate(rate): Updates the continuous power consumption rate (kW).

- scheduleSpike(time, energy): Schedules a one-time power draw event at specified simulation time (hr) with given energy requirement (kWh).

- getCurrentPowerDemand(dt): Calculates total power demand for time period dt. Automatically processes any spike events and updates totalEnergyConsumed.

LunarRover

Initialization

- system: SimPy environment instance for the simulation

- name: Identifier string for the rover

- roverType: Type designation - either 'crew' or 'cargo'

- maxCapacity: Maximum cargo carrying capacity (kg)

- energyPerKm: Energy consumption rate per kilometer traveled (kWh/km)

- batteryCapacity: Maximum battery energy storage (kWh)

Additional Notable Object Variables

- type: Rover type ('crew' or 'cargo')

- currentLoad: Current mass of cargo being carried (kg)

- batteryCharge: Current battery charge level (kWh)

- totalDistanceTraveled: Cumulative distance traveled since start (km)

- totalEnergyConsumed: Cumulative energy used for travel (kWh)

Functions

- travel(distance, travelTime): SimPy process that simulates traveling a distance (km) over a time period (hr). Consumes battery energy and yields for travel duration. Raises RuntimeError if insufficient battery.

- loadCargo(mass): Immediately adds cargo mass (kg) to currentLoad. Raises ValueError if exceeds maxCapacity.

- unloadCargo(): Immediately removes all cargo and returns the unloaded mass (kg). Resets currentLoad to 0.

RoverChargingStation

Initialization

- system: SimPy environment instance for the simulation

- name: Identifier string for the charging station

- chargingPowerRate: Power consumption and charging rate (kW)

- efficiencyFactor: Charging efficiency accounting for losses (0-1), default=0.9

Additional Notable Object Variables

- totalEnergyConsumed: Cumulative energy drawn from power grid (kWh)

- totalEnergyDelivered: Cumulative energy actually delivered to rovers (kWh)

- resource: SimPy Resource limiting to one rover charging at a time

Functions

- chargeRover(rover): SimPy process that fully charges a rover's battery. Requests the charging resource, calculates charging time based on power rate, yields for that duration, then restores rover battery to full capacity.

- getCurrentPowerDemand(dt): Returns power demand for time period dt. Returns chargingPowerRate * dt if station is in use, otherwise 0.

LandingLaunchZone

Initialization

- system: SimPy environment instance for the simulation

- name: Identifier string for the landing/launch facility

- loxCapacity: Maximum LOX storage capacity (kg)

- loxChillingPowerRate: Continuous power required to keep LOX chilled (kW)

- utilitiesPowerRate: Continuous power for lighting and other utilities (kW)

Additional Notable Object Variables

- loxStored: Current amount of LOX in storage tanks (kg)

- totalEnergyConsumed: Cumulative energy used by the facility (kWh)

- spikeEvents: List of scheduled one-time power events as (time, energy) tuples

Functions

- receiveLOX(amount): Immediately adds LOX mass (kg) to storage. Raises ValueError if exceeds loxCapacity.

- consumeLOX(amount): Immediately removes and returns LOX mass (kg) from storage. Raises ValueError if insufficient LOX stored.

- scheduleSpike(time, energy): Schedules a one-time power draw event at specified simulation time (hr) with given energy requirement (kWh).

- getCurrentPowerDemand(dt): Calculates total power demand for time period dt (base chilling + utilities + any spike events). Updates totalEnergyConsumed.